Two-Wheeler Behaviour Recognition
Final Independent-Test Model Report

Fixed protocol: seven raw features, 156-sample windows, stride 78, train/validation selection, combined retraining, and one held-out test evaluation.

# Summary

27 model entries were evaluated. The best independent-test accuracy is 74.64% (histgradientboosting). No model reaches 75% under this completed strict protocol.

# Independent-test comparison

| Rank | Model | Accuracy | Macro-F1 |
| --- | --- | --- | --- |
| 1 | histgradientboosting | 74.64% | 73.74% |
| 2 | lightgbm | 74.55% | 72.94% |
| 3 | rbf_svm | 72.63% | 70.45% |
| 4 | xgboost | 71.87% | 69.44% |
| 5 | mlp_tabular | 71.48% | 68.56% |
| 6 | tcn | 69.09% | 68.37% |
| 7 | random_forest | 68.71% | 65.70% |
| 8 | lstm_attention | 67.75% | 66.52% |
| 9 | fusformer | 67.27% | 65.95% |
| 10 | mega | 66.99% | 65.96% |
| 11 | linear_svm | 66.99% | 65.84% |
| 12 | bilstm_attention | 66.70% | 65.78% |
| 13 | gru | 66.32% | 65.33% |
| 14 | bilstm | 64.98% | 64.36% |
| 15 | conv_transformer | 64.88% | 63.74% |
| 16 | tcnca_v1 | 64.59% | 64.51% |
| 17 | convnet_har | 64.02% | 63.70% |
| 18 | tst | 63.25% | 63.43% |
| 19 | tcnca_v2 | 62.30% | 62.29% |
| 20 | logistic_regression | 60.96% | 61.94% |
| 21 | lstm | 58.47% | 58.32% |
| 22 | tcnca_v3 | 57.42% | 57.94% |
| 23 | s4d | 51.20% | 44.91% |
| 24 | s5_onnx | 51.20% | 44.91% |
| 25 | lru | 51.20% | 44.91% |
| 26 | resnet1d | 38.47% | 36.03% |
| 27 | sgconv | 33.78% | 26.44% |

# Protocol and limitations

Every model uses the same seven raw channels, split files, and windowing. Model-specific internal architectures are allowed, but test labels are not used for selection. The validation/test gap indicates distribution shift; macro-F1 is included because the classes are imbalanced. Raspberry Pi 5 latency and memory still require hardware measurement.

Data: train.csv, val.csv, train_val_combined.csv, test.csv. Features: Ax, Ay, Az, Gx, Gy, Gz, Speed. Classes: BUMP, LEFT, RIGHT, STOP, STRAIGHT.