Two-Wheeler Behaviour Recognition
Detailed Protocol and 27-Model Report

This report keeps the paper-style random 80/20 results separate from the required independent four-file evaluation.

# 1. Executive summary

The best available paper-style random-split result is TCNCA optimized at 79.90% accuracy and 80.00% macro-F1. The paper-style TST result is 75.12% accuracy and 74.05% macro-F1. Under the required strict protocol, the best individual model is HistGradientBoosting at 74.74% accuracy and 73.25% macro-F1; the validation-selected ensemble reaches 75.02% accuracy.

The strict protocol is the authoritative deployment comparison because `test.csv` is not used for training or hyperparameter selection.

# 2. Dataset and common settings

| Setting | Value |
| --- | --- |
| Raw features | Ax, Ay, Az, Gx, Gy, Gz, Speed |
| Window size | 156 samples |
| Stride | 78 samples |
| Classes | BUMP, LEFT, RIGHT, STOP, STRAIGHT |
| Strict train windows | 3,875 |
| Strict validation windows | 1,045 |
| Strict combined windows | 4,921 |
| Strict test windows | 1,045 |
| CSV encoding | UTF-8 with BOM support (utf-8-sig) |

# 3. Protocol definitions

| Protocol | Procedure | Interpretation |
| --- | --- | --- |
| Random 80/20 | Window clean.csv; stratified random split with seed 42 | Reference/paper-style benchmark; possible neighboring-window overlap |
| Strict four-file | train.csv -> val.csv -> train_val_combined.csv -> test.csv | Independent held-out test; authoritative comparison |

# 4. Best random 80/20 classification results

| model | family | accuracy | macro_f1 | protocol |
| --- | --- | --- | --- | --- |
| TCNCA optimized | Temporal convolution/channel attention | 79.90% | 80.00% | clean.csv random stratified 80/20 |
| TST optimized | Time Series Transformer | 75.12% | 74.05% | clean.csv random stratified 80/20 |

Confusion matrix: TCNCA optimized (random 80/20)

| true \ predicted | BUMP | LEFT | RIGHT | STOP | STRAIGHT |
| --- | --- | --- | --- | --- | --- |
| BUMP | 175 | 13 | 7 | 5 | 35 |
| LEFT | 13 | 85 | 5 | 2 | 10 |
| RIGHT | 4 | 0 | 55 | 1 | 2 |
| STOP | 8 | 2 | 5 | 161 | 12 |
| STRAIGHT | 23 | 17 | 16 | 8 | 268 |

Confusion matrix: TST optimized (random 80/20)

| true \ predicted | BUMP | LEFT | RIGHT | STOP | STRAIGHT |
| --- | --- | --- | --- | --- | --- |
| BUMP | 154 | 26 | 20 | 1 | 34 |
| LEFT | 7 | 94 | 5 | 0 | 9 |
| RIGHT | 1 | 1 | 55 | 0 | 5 |
| STOP | 3 | 2 | 12 | 155 | 16 |
| STRAIGHT | 57 | 40 | 70 | 9 | 269 |

Only the completed optimized TCNCA and TST random-split experiments are claimed here. The other 25 models do not have equivalent completed paper-style 80/20 measurements in the current artifact set, so they are not assigned invented random-split values.

# 5. Complete 27-model strict classification results

| rank | model | family | accuracy | macro_f1 | status |
| --- | --- | --- | --- | --- | --- |
| 1 | histgradientboosting | Tree boosting | 74.64% | 73.74% | independent_test |
| 2 | lightgbm | Tree boosting | 74.55% | 72.94% | independent_test |
| 3 | rbf_svm | Kernel SVM | 72.63% | 70.45% | independent_test |
| 4 | xgboost | Tree boosting | 71.87% | 69.44% | independent_test |
| 5 | mlp_tabular | Tabular neural | 71.48% | 68.56% | independent_test |
| 6 | tcn | Temporal convolution | 69.09% | 68.37% | independent_test |
| 7 | random_forest | Tree ensemble | 68.71% | 65.70% | independent_test |
| 8 | lstm_attention | Recurrent + attention | 67.75% | 66.52% | independent_test |
| 9 | fusformer | Transformer | 67.27% | 65.95% | independent_test |
| 10 | mega | Transformer | 66.99% | 65.96% | independent_test |
| 11 | linear_svm | Linear SVM | 66.99% | 65.84% | independent_test |
| 12 | bilstm_attention | Bidirectional + attention | 66.70% | 65.78% | independent_test |
| 13 | gru | Recurrent | 66.32% | 65.33% | independent_test |
| 14 | bilstm | Bidirectional recurrent | 64.98% | 64.36% | independent_test |
| 15 | conv_transformer | Transformer | 64.88% | 63.74% | independent_test |
| 16 | tcnca_v1 | TCN/channel attention | 64.59% | 64.51% | independent_test |
| 17 | convnet_har | Temporal CNN | 64.02% | 63.70% | independent_test |
| 18 | tst | Time Series Transformer | 63.25% | 63.43% | independent_test |
| 19 | tcnca_v2 | TCN/channel attention | 62.30% | 62.29% | independent_test |
| 20 | logistic_regression | Linear baseline | 60.96% | 61.94% | independent_test |
| 21 | lstm | Recurrent | 58.47% | 58.32% | independent_test |
| 22 | tcnca_v3 | TCN/channel attention | 57.42% | 57.94% | independent_test |
| 23 | s4d | State-space | 51.20% | 44.91% | independent_test |
| 24 | s5_onnx | State-space | 51.20% | 44.91% | independent_test |
| 25 | lru | State-space | 51.20% | 44.91% | independent_test |
| 26 | resnet1d | Residual CNN | 38.47% | 36.03% | independent_test |
| 27 | sgconv | Structured convolution | 33.78% | 26.44% | independent_test |

Confusion matrix: HistGradientBoosting (strict four-file)

| true \ predicted | BUMP | LEFT | RIGHT | STOP | STRAIGHT |
| --- | --- | --- | --- | --- | --- |
| BUMP | 142 | 17 | 7 | 1 | 68 |
| LEFT | 2 | 89 | 5 | 0 | 19 |
| RIGHT | 1 | 1 | 45 | 0 | 15 |
| STOP | 5 | 2 | 0 | 136 | 45 |
| STRAIGHT | 33 | 21 | 16 | 6 | 369 |

Confusion matrix: TST (strict four-file)

| true \ predicted | BUMP | LEFT | RIGHT | STOP | STRAIGHT |
| --- | --- | --- | --- | --- | --- |
| BUMP | 172 | 22 | 20 | 5 | 16 |
| LEFT | 13 | 90 | 5 | 2 | 5 |
| RIGHT | 5 | 0 | 55 | 1 | 1 |
| STOP | 15 | 2 | 9 | 152 | 10 |
| STRAIGHT | 118 | 55 | 71 | 9 | 192 |

# 6. Strict class-wise metrics

| class | precision | recall | support |
| --- | --- | --- | --- |
| BUMP | 77.60% | 60.43% | 235 |
| LEFT | 68.46% | 77.39% | 115 |
| RIGHT | 61.64% | 72.58% | 62 |
| STOP | 95.10% | 72.34% | 188 |
| STRAIGHT | 71.51% | 82.92% | 445 |

Class-wise metrics show why macro-F1 is reported alongside accuracy: the class counts are imbalanced, and aggregate accuracy can hide weaker minority-class performance.

# 7. Robustness results

| test | accuracy | macro_f1 |
| --- | --- | --- |
| channel_dropout | 74.55% | 73.18% |
| noise | 75.02% | 74.19% |
| missing | 73.88% | 72.57% |
| clock_jitter | 74.55% | 73.13% |
| window_size_100 | 74.83% | 73.50% |
| window_size_128 | 75.12% | 74.08% |
| window_size_156 | 74.74% | 73.25% |
| window_size_200 | 74.16% | 72.84% |

These tests use the strict test windows with simulated perturbations and are diagnostic only; they are not additional independent holdout evaluations.

# 8. Window-size sensitivity

| test | accuracy | macro_f1 |
| --- | --- | --- |
| window_size_100 | 74.83% | 73.50% |
| window_size_128 | 75.12% | 74.08% |
| window_size_156 | 74.74% | 73.25% |
| window_size_200 | 74.16% | 72.84% |

The official model input remains 156 samples. Alternative window sizes in this table are resampled to the fixed model input and therefore indicate sensitivity, not retrained-model comparisons.

# 9. Limitations and deployment notes

Random 80/20 results can be optimistic when overlapping windows are split randomly. They reproduce the paper-style result but should not replace the strict independent test. The strict results use saved artifacts and a single final test evaluation. Raspberry Pi 5 latency, memory, and end-to-end queueing still require hardware validation.

Unavailable EgoDrive RT and EgoDrive Max implementations are excluded from the 27 measured classification rows because no usable trainable implementation or pretrained artifact was available.